Matching list Cat 3508B-1200RPM – Mekanord gearbox – Helseth propeller.

Selection Guide – open propeller.

| Engine type | Type of gearbox & propeller | Engine RPM | Engine power kW/HP | Propeller RPM | Red. ratio | Propeller dia. mm | Hub dia mm | Weigth gearbox Kg. | Weigth propeller Kg. | Length engine mm | Height engine mm | Width engine mm | Weigth engine Kg. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3508B | 400HS 4T410/140 | 1200 | 578/775 | 336 | 3,57:1 | 1850 | 410 | 900 |  | 2077 | 1822 | 1703 | 5500 |
|  | 400HS 4T450/160 | 1200 | 578/775 | 258 | 4,56:1 | 2100 | 450 | 900 |  |  |  |  |  |
|  | 430-1HS 4TX550/170 | 1200 | 578/775 | 204 | 5,87:1 | 2400 | 550 | 1380 |  |  |  |  |  |
| # Selection Guide – nozzle propeller |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3508B | 400HS 4T410/140 | 1200 | 578/775 | 336 | 3,57:1 | 1700 | 410 | 900 |  | 2077 | 1822 | 1703 | 5500 |
|  | 400HS 4T450/160 | 1200 | 578/775 | 258 | 4,65:1 | 1900 | 450 | 900 |  |  |  |  |  |
|  | 430-1HS 4TX550/170 | 1200 | 578/775 | 204 | 5,87:1 | 2200 | 550 | 1380 |  |  |  |  |  |

Matching list Cat 3508B-1600RPM – Mekanord gearbox – Helseth propeller.

Selection Guide – open propeller.

| Engine type | Type of gearbox & propeller | Engine RPM | Engine power kW/HP | Propeller RPM | Red. ratio | Propeller dia. mm | Hub dia mm | Weigth gearbox Kg. | Weigth propeller Kg. | Length engine mm | Height engine mm | Width engine mm | Weigth engine Kg. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3508B | 430-1HS 4T450/160 | 1600 | 746/1000 | 317 | 5,04:1 | 2000 | 450 | 1380 |  | 2077 | 1822 | 1703 | 5500 |
|  | 430-1HS 4T450/170 | 1600 | 746/1000 | 273 | 5,87:1 | 2150 | 450 | 1380 |  |  |  |  |  |
|  | 500/80HS 4TX550/170 | 1600 | 746/1000 | 244 | 6,56:1 | 2350 | 550 | 2400 |  |  |  |  |  |
| # Selection Guide – nozzle propeller |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3508B | 430-1HS 4T450/160 | 1600 | 746/1000 | 317 | 5,04:1 | 1850 | 450 | 1380 |  | 2077 | 1822 | 1703 | 5500 |
|  | 430-1HS 4T450/170 | 1600 | 746/1000 | 273 | 5,87:1 | 2000 | 450 | 1380 |  |  |  |  |  |
|  | 500/80HS 4TX550/170 | 1600 | 746/1000 | 245 | 6,56:1 | 2100 | 550 | 2400 |  |  |  |  |  |

Matching list Cat 3512B – Mekanord gearbox – Helseth propeller.

Selection Guide – open propeller.

| Engine type | Type of gearbox & propeller | Engine RPM | Engine power kW/HP | Propeller RPM | Red. ratio | Propeller dia. mm | Hub dia mm | Weigth gearbox Kg. | Weigth propeller Kg. | Length engine mm | Height engine mm | Width engine mm | Weigth engine Kg. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3512B | 500/80HS 4TX550/180 | 1200 | 970/1300 | 263 | 4,56:1 | 2350 | 550 | 2400 |  | 3149 | 2105 | 1941 | 6800 |
|  | 500/80HS 4TX550/180 | 1200 | 970/1300 | 243 | 4;94:1 | 2400 | 550 | 2400 |  |  |  |  |  |
|  | 500/80HS 4TX550/190 | 1200 | 970/1300 | 212 | 5,67:1 | 2600 | 550 | 2400 |  |  |  |  |  |
| # Selection Guide – nozzle propeller |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3512B | 500/80HS 4TX550/180 | 1200 | 970/1300 | 263 | 4,56:1 | 2150 | 550 | 2400 |  | 3149 | 2105 | 1941 | 6800 |
|  | 500/80HS 4TX550/180 | 1200 | 970/1300 | 243 | 4,94:1 | 2250 | 550 | 2400 |  |  |  |  |  |
|  | 500/80HS 4TX550/190 | 1200 | 970/1300 | 212 | 5,67:1 | 2350 | 550 | 2400 |  |  |  |  |  |

Matching list Cat 3512B-HD – Mekanord gearbox – Helseth propeller.

Selection Guide – open propeller.

| Engine type | Type of gearbox & propeller | Engine RPM | Engine power kW/HP | Propeller RPM | Red. ratio | Propeller dia. mm | Hub dia mm | Weigth gearbox Kg. | Weigth propeller Kg. | Length engine mm | Height engine mm | Width engine mm | Weigth engine Kg. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3512B-HD | 500HS 4TX550/190 | 1200 | 1119/1500 | 263 | 4,56:1 | 2400 | 550 | 2400 |  | 3149 | 2105 | 1941 | 6800 |
|  | 500HS 4TX550/190 | 1200 | 1119/1500 | 243 | 4;94:1 | 2500 | 550 | 2400 |  |  |  |  |  |
|  | 500HS 4TX650/200 | 1200 | 1119/1500 | 212 | 5,67:1 | 2700 | 650 | 2400 |  |  |  |  |  |
| # Selection Guide – nozzle propeller |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3512B-HD | 500HS 4TX550/190 | 1200 | 1119/1500 | 263 | 4,56:1 | 2200 | 550 | 2400 |  | 3149 | 2105 | 1941 | 6800 |
|  | 500HS 4TX550/190 | 1200 | 1119/1500 | 243 | 4,94:1 | 2300 | 550 | 2400 |  |  |  |  |  |
|  | 500HS 4TX650/200 | 1200 | 1119/1500 | 212 | 5,67:1 | 2450 | 650 | 2400 |  |  |  |  |  |

Matching list Cat 3516B – Mekanord gearbox – Helseth propeller.

Selection Guide – open propeller.

| Engine type | Type of gearbox & propeller | Engine RPM | Engine power kW/HP | Propeller RPM | Red. ratio | Propeller dia. mm | Hub dia mm | Weigth gearbox Kg. | Weigth propeller Kg. | Length engine mm | Height engine mm | Width engine mm | Weigth engine Kg. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3516B | 500HS 4TX550/180 | 1200 | 1231/1650 | 296 | 4,05:1 | 2350 | 550 | 2400 |  | 3689 | 2110 | 2144 | 8500 |
|  | 500HS 4TX650/200 | 1200 | 1231/1650 | 243 | 4;94:1 | 2600 | 650 | 2400 |  |  |  |  |  |
|  | 580HS 4TX650/220 | 1200 | 1231/1650 | 185 | 6,47:1 | 3000 | 650 | 4900 |  |  |  |  |  |
| # Selection Guide – nozzle propeller |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3516B | 500HS 4TX550/180 | 1200 | 1231/1650 | 296 | 4,05:1 | 2150 | 550 | 2400 |  | 3689 | 2110 | 2144 | 8500 |
|  | 500HS 4TX650/200 | 1200 | 1231/1650 | 243 | 4,94:1 | 2400 | 650 | 2400 |  |  |  |  |  |
|  | 580HS 4TX650/230 | 1200 | 1231/1650 | 185 | 6,47:1 | 2650 | 650 | 4900 |  |  |  |  |  |

Matching list Cat 3516B-HD – Mekanord gearbox – Helseth propeller.

Selection Guide – open propeller.

| Engine type | Type of gearbox & propeller | Engine RPM | Engine power kW/HP | Propeller RPM | Red. ratio | Propeller dia. mm | Hub dia mm | Weigth gearbox Kg. | Weigth propeller Kg. | Length engine mm | Height engine mm | Width engine mm | Weigth engine Kg. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3516B-HD | 500HS 4TX550/190 | 1200 | 1398/1875 | 296 | 4,05:1 | 2400 | 550 | 2400 |  | 3689 | 2110 | 2144 | 8500 |
|  | 500HS 4TX650/220 | 1200 | 1398/1875 | 243 | 4;94:1 | 2650 | 650 | 2400 |  |  |  |  |  |
|  | 580HS 4TX750/230 | 1200 | 1398/1875 | 185 | 6,47:1 | 3100 | 750 | 4900 |  |  |  |  |  |
| # Selection Guide – nozzle propeller |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3516B-HD | 500HS 4TX550/190 | 1200 | 1398/1875 | 296 | 4,05:1 | 2200 | 550 | 2400 |  | 3689 | 2110 | 2144 | 8500 |
|  | 500HS 4TX650/220 | 1200 | 1398/1875 | 243 | 4,94:1 | 2450 | 650 | 2400 |  |  |  |  |  |
|  | 580HS 4TX750/230 | 1200 | 1398/1875 | 185 | 6,47:1 | 2800 | 750 | 4900 |  |  |  |  |  |