Matchinglist MAN propeller.

| Engine Power Output 100% MCR at 1000 RPM | Propeller hub | Propeller Design speed RPM | Propeller diameter mm | MEP Std. gearbox | MEP Hybrid gearbox | Reduction ratio | Propeller Actual speed RPM |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1290 kW | VBS540 VBS600 VBS660 VBS660 | 272 231 203 179 | 2350 2600 2800 3000 | 500OD/HS 500OD/HS 500OD/HS 580OD/HS | Type50OD/HS Type50OD/HS Type50OD/HS Type60OD/HS | 3,52:1 4,24:1 4,94:1 5,65:1 | 284 235 202 177 |
| 1505 kW | VBS600 VBS660 VBS660 VBS660 | 258 222 196 175 | 2500 2750 2950 3150 | 500OD/HS 580OD/HS 580OD/HS 580OD/HS | Type50OD/HS Type60OD/HS Type60OD/HS Type60OD/HS | 4,24:1 4,53:1 5,06:1 5,65:1 | 235 221 198 177 |
| 1720kW | VBS660 VBS660 VBS660 VBS720 | 261 219 195 174 | 2550 2850 3050 3250 | 500OD/HS 580OD/HS 580OD/HS 580OD/HS | Type50OD/HS Type60OD/HS Type60OD/HS Type60OD/HS | 3,63:1 4,53:1 5,06:1 5,65:1 | 275 221 198 177 |
| 1935kW | VBS660 VBS660 VBS720 VBS720 | 262 221 198 187 | 2600 2900 3100 3200 | 500OD/HS 580OD/HS 580OD/HS 580OD/HS | Type50OD/HS Type60OD/HS Type60OD/HS Type60OD/HS | 3,63:1 4,53:1 5,06:1 5,65:1 | 275 221 198 177 |

Above is indicated some possible combinations, but please observe, that many other solutions are possible.

All mentioned std. gearboxes can as option be delivered with up to 3 PTO’s, special PTI and stand-by pump.