| Engine |  | Gearbox |  | Propeller |  |  |  |  |  |  |  | Prices in DKK |  |  |  |  | Remarks |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HP | RPM | Type | Red.ratio |  |  | Free prop. |  | Nozzle Prop. |  |  | Gearbox |  |  | Propeller |  |  |  |  |
|  |  |  |  | RPM | Size | Dia. | Bollard Pull |  | Dia. | Bollard pull |  |  |  |  |  |  |  |  |
| 937 | 1600 | 400HS | 4,11:1 | 389 | 10 | 1,81 | 9,6 |  | 1,63 | 12,1 |  |  | 258.300 |  | 248.000 |  |  |  |
| 937 | 1600 | 400HS | 4,65:1 | 344 | 10 | 1,95 | 10,0 |  | 1,73 | 12,6 |  |  | 258 300 |  | 248 000 |  |  |  |
| 937 | 1600 | 400HS | 5,17:1 | 309 | 10½S | 2,08 | 10,5 |  | 1,82 | 13,0 |  |  | 258 300 |  | 371 000 |  |  |  |
| 937 | 1600 | 400HS | 5,53:1 | 289 | 10½S | 2,16 | 10,8 |  | 1,89 | 13,3 |  |  | 258 300 |  | 371 000 |  |  |  |
| 937 | 1600 | 430HS | 5,87:1 | 273 | 10½S | 2,24 | 11,0 |  | 1,94 | 13,6 |  |  | 302 400 |  | 371 000 |  |  |  |
| 937 | 1600 | 430HS | 6,53:1 | 245 | 10½S | 2,39 | 11,5 |  | 2,05 | 14,1 |  |  | 302 400 |  | 371 000 |  |  |  |
| 1034 | 1650 | 430HS | 4,15:1 | 398 | 10 | 1,82 | 10,2 |  | 1,65 | 13,0 |  |  | 302 400 |  | 248 000 |  |  |  |
| 1034 | 1650 | 430HS | 4,56:1 | 362 | 10 | 1,93 | 10,6 |  | 1,73 | 13,4 |  |  | 302 400 |  | 248 000 |  |  |  |
| 1034 | 1650 | 430HS | 5,04:1 | 327 | 10½S | 2,05 | 11,1 |  | 1,82 | 13,9 |  |  | 302 400 |  | 371 000 |  |  |  |
| 1034 | 1650 | 430HS | 5,87:1 | 281 | 10½S | 2,24 | 11,8 |  | 1,96 | 14,6 |  |  | 302 400 |  | 371 000 |  |  |  |
| 1034 | 1650 | 500 HS/LD | 6,56:1 | 252 | 10½S | 2,40 | 12,3 |  | 2,07 | 15,2 |  |  | 430 000 |  | 371 000 |  |  |  |
| 1059 | 1800 | 400HS | 4,11:1 | 438 | 10 | 1,73 | 10,0 |  | 1,58 | 12,9 |  |  | 258 300 |  | 248 000 |  |  |  |
| 1059 | 1800 | 400HS | 4,65:1 | 387 | 10 | 1,86 | 10,6 |  | 1,68 | 13,4 |  |  | 258 300 |  | 248 000 |  |  |  |
| 1059 | 1800 | 400HS | 5,17:1 | 348 | 10½S | 1,98 | 11,0 |  | 1,77 | 13,9 |  |  | 258 300 |  | 371 000 |  |  |  |
| 1059 | 1800 | 400HS | 5,53:1 | 325 | 10½S | 2,07 | 11,3 |  | 1,83 | 14,2 |  |  | 258 300 |  | 371 000 |  |  |  |
| 1059 | 1800 | 430HS | 5,87:1 | 307 | 10½S | 2,14 | 11,6 |  | 1,89 | 14,5 |  |  | 302 400 |  | 371 000 |  |  |  |
| 1059 | 1800 | 430HS | 6,53:1 | 276 | 10½S | 2,28 | 12,1 |  | 1,99 | 15,0 |  |  | 302 400 |  | 371 000 |  |  |  |
| 1250 | 1600 | 430/80 HS | 4,15:1 | 386 | 10½S | 1,93 | 12,1 |  | 1,76 | 15,4 |  |  | 372 800 |  | 371 000 |  |  |  |
| 1250 | 1600 | 430/80 HS | 5,12:1 | 313 | 10½S | 2,19 | 13,1 |  | 1,95 | 16,5 |  |  | 372 800 |  | 371 000 |  |  |  |
| 1250 | 1600 | 500 HS/LD | 5,67:1 | 282 | 11 | 2,33 | 13,7 |  | 2,05 | 17,1 |  |  | 430 000 |  | 466 000 |  |  |  |
| 1250 | 1600 | 500 HS/LD | 6,56:1 | 244 | 11 | 2,54 | 14,5 |  | 2,21 | 18,0 |  |  | 430 000 |  | 466 000 |  |  |  |
| 1376 | 1650 | 500 HS/LD | 4,05:1 | 407 | 10½S | 1,90 | 12,8 |  | 1,75 | 16,4 |  |  | 430 000 |  | 371 000 |  |  |  |
| 1376 | 1650 | 500 HS/LD | 4,94:1 | 334 | 10½S | 2,14 | 13,8 |  | 1,93 | 17,5 |  |  | 430 000 |  | 371 000 |  |  |  |
| 1376 | 1650 | 500 HS/LD | 5,67:1 | 291 | 11 | 2,33 | 14,6 |  | 2,07 | 18,3 |  |  | 430 000 |  | 466 000 |  |  |  |
| 1376 | 1650 | 500 HS/LD | 6,56:1 | 252 | 11 | 2,54 | 15,5 |  | 2,23 | 19,3 |  |  | 430 000 |  | 466 000 |  |  |  |
| 1413 | 1800 | 430/80 HS | 4,15:1 | 434 | 10½S | 1,84 | 12,7 |  | 1,71 | 16,4 |  |  | 372 800 |  | 371 000 |  |  |  |
| 1413 | 1800 | 500 HS/LD | 4,94:1 | 364 | 10½S | 2,04 | 13,6 |  | 1,86 | 17,4 |  |  | 430 000 |  | 371 000 |  |  |  |
| 1413 | 1800 | 500 HS/LD | 5,67:1 | 317 | 11 | 2,22 | 14,4 |  | 2,00 | 18,2 |  |  | 430 000 |  | 466 000 |  |  |  |
| 1413 | 1800 | 500 HS/LD | 6,56:1 | 274 | 11 | 2,42 | 15,3 |  | 2,15 | 19,1 |  |  | 430 000 |  | 466 000 |  |  |  |

| 1875 | 1600 | 500HS | 4,05:1 | 395 | 11 | 2,06 | 16,5 | 1,92 | 21,4 | 470 000 | 466 000 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1875 | 1600 | 500HS | 4,94:1 | 324 | 11 | 2,32 | 17,9 | 2,12 | 22,9 | 470 000 | 466 000 |  |
| 1875 | 1600 | 500HS | 5,67:1 | 282 | 11½ | 2,52 | 18,9 | 2,27 | 24,0 | 470 000 | 614 000 |  |
| 2065 | 1650 | 500HS | 4,05:1 | 407 | 11 | 2,06 | 17,7 | 1,94 | 23,0 | 470 000 | 466 000 |  |
| 2065 | 1650 | 500HS | 4,94:1 | 334 | 11 | 2,32 | 19,1 | 2,14 | 24,6 | 470 000 | 466 000 |  |
| 2065 | 1650 | 500HS | 5,67:1 | 291 | 11½ | 2,52 | 20,2 | 2,29 | 25,7 | 470 000 | 614 000 |  |
| 2119 | 1800 | 500HS | 4,05:1 | 444 | 11 | 1,97 | 17,4 | 1,87 | 22,8 | 470 000 | 466 000 |  |
| 2119 | 1800 | 500HS | 4,94:1 | 364 | 11 | 2,22 | 18,8 | 2,06 | 24,4 | 470 000 | 466 000 |  |
| 2119 | 1800 | 500HS | 5,67:1 | 317 | 11½ | 2,41 | 19,9 | 2,21 | 25,5 | 470 000 | 614 000 |  |
| 895 | 1000 | 430/80HS | 4,15:1 | 241 | 10½S | 2,39 | 11,2 | 2,04 | 13,6 | 372 800 | 371 000 |  |
| 895 | 1000 | 500 HS/LD | 4,94:1 | 202 | 11 | 2,66 | 12,0 | 2,23 | 14,5 | 430 000 | 466 000 |  |
| 895 | 1000 | 500 HS/LD | 5,67:1 | 176 | 11½ * | 2,88 | 12,6 | 2,39 | 15,1 | 430 000 | 614 000 |  |
| 895 | 1000 | 500 HS/LD | 6,56:1 | 152 | 11½ * | 3,15 | 13,4 | 2,57 | 15,9 | 430 000 | 614 000 |  |
| 843 | 900 | 430/80 HS | 3,15:1 | 286 | 10½S * | 2,13 | 9,9 | 1,85 | 12,3 | 372 800 | 371 000 |  |
| 843 | 900 | 430/80 HS | 4,15:1 | 217 | 10½S | 2,52 | 11,1 | 2,12 | 13,4 | 372 800 | 371 000 |  |
| 843 | 900 | 500 HS/LD | 4,94:1 | 182 | 11½ * | 2,79 | 11,9 | 2,32 | 14,2 | 430 000 | 614 000 |  |
| 843 | 900 | 500 HS/LD | 5,67:1 | 159 | 11½ * | 3,04 | 12,6 | 2,48 | 14,9 | 430 000 | 614 000 |  |
| 734 | 750 | 430/80 HSX | 3,12:1 | 240 | 10½S | 2,30 | 9,5 | 1,95 | 11,6 | 385 000 | 371 000 |  |
| 734 | 750 | 430/80 HSX | 3,52:1 | 213 | 10½S | 2,47 | 10,0 | 2,07 | 12,0 | 385 000 | 371 000 |  |
| 734 | 750 | 430/80HSX | 4,15:1 | 181 | 10½S | 2,73 | 10,7 | 2,25 | 12,7 | 385 000 | 371 000 |  |
| 734 | 750 | 500 HS/LD | 4,94:1 | 152 | 11½ * | 3,03 | 11,5 | 2,45 | 13,5 | 430 000 | 614 000 |  |
| 1346 | 1000 | 500HS | 3,04:1 | 329 | 10½S | 2,15 | 13,7 | 1,94 | 17,3 | 470 000 | 371 000 |  |
| 1346 | 1000 | 500HS | 4,05:1 | 247 | 11 | 2,56 | 15,3 | 2,24 | 19,0 | 470 000 | 466 000 |  |
| 1346 | 1000 | 500HS | 4,94:1 | 202 | 11½ | 2,88 | 16,6 | 2,47 | 20,3 | 470 000 | 614 000 |  |
| 1265 | 900 | 500HS | 3,04:1 | 296 | 10½S | 2,27 | 13,6 | 2,01 | 17,0 | 470 000 | 371 000 |  |
| 1265 | 900 | 500HS | 4,05:1 | 222 | 11 | 2,69 | 15,2 | 2,32 | 18,7 | 470 000 | 466 000 |  |
| 1265 | 900 | 500HS | 4,94:1 | 182 | 11½ | 3,03 | 16,5 | 2,56 | 20,0 | 470 000 | 614 000 |  |
| 1101 | 750 | 500HS | 3,04:1 | 247 | 11 | 2,46 | 13,0 | 2,13 | 16,1 | 470 000 | 466 000 |  |
| 1101 | 750 | 500HS | 4,05:1 | 185 | 11½ * | 2,92 | 14,6 | 2,46 | 17,7 | 470 000 | 614 000 |  |
| 1101 | 750 | 500HS | 4,94:1 | 152 | 12 * | 3,29 | 15,8 | 2,71 | 18,9 | 470 000 | 725 000 |  |

The matchinglist indicates a number of possibly combinations, which must however only be understood as indicative. Specially for the cases marked with “*”, we suggest you to consulate with us for the specific project to find the best solution.

Dia shown is the optimum dia, and Bollard Pull in the calculated theoretical Bollard Pull. Prices are gross prices for standard equipment.