mekanord C.P.-propulsion 	Matching list for Wartsila, Mulhouse-engines	26.10.00

| # Engine |  |  |  | # Gearbox |  | # C.P-propeller |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | # Open |  | # In Nozzle |  |
| Type | RPM | kW/HP | kW/RPM | Type | Red. ratio | RPM | #### Type | Diameter (m) | Bollard pull* (Tons) | Diameter (m) | Bollard pull* ### (Tons) |
| ##### 12V200 | 1200 | 2100/2850 | 1,75 | 500/120HS | 3,63:1 | 331 | VP11½ | 2,45 | 24,5 | 2,41 | 33,0 |
|  | 1200 | 2100/2850 | 1,75 | 580HS | 4,53:1 | 265 | VP12 | 2,80 | 26,8 | 2,69 | 35,5 |
| ##### 16V200 | 1200 | 2800/3800 | 2,33 | ###### 580HS | 4,53:1 | 265 | VP14 | 2,96 | 33,8 | 2,89 | 45,1 |
| ##### 18V200 | 1200 | 3150/4280 | 2,63 | 580HS | 4,53:1 | 265 | VP14 | 3,06 | 37,1 | 2,98 | 49,9 |
| ##### 12V200 | 1500 | 2400/3260 | 1,6 | 580HS | 5,65:1 | 265 | VP12 | 2,87 | 29,8 | 2,78 | 39,7 |
| ##### 16V200 | 1500 | 3200/4350 | 2,13 | 650HS | 5,44:1 | 276 | VP14 | 2,97 | 37,0 | 2,93 | 49,9 |
| ##### 18V200 | 1500 | 3600/4890 | 2,4 | 650HS | 5,05:1 | 287 | VP14 | 2,91 | 39,5 | 2,91 | 53,6 |

Above is some indicative combinations, but please observe, that many other solutions are possible.

mekanord: Phone: + 45 96 20 14 00,  Fax: + 45 96 20 14 14,  e-mail:  mekanord@mekanord.dk                                                  * = Theoretical

mekanord C.P.-propulsion 	Matching list for Wartsila -engines	26.10.00

| # Engine |  |  |  | # Gearbox |  | # C.P-propeller |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | # Open |  | # In Nozzle |  |
| Type | RPM | kW/HP | kW/RPM | Type | Red. ratio | RPM | #### Type | Diameter (m) | Bollard pull* (Tons) | Diameter (m) | Bollard pull* ### (Tons) |
| ##### 4L20 | 1000 | 660/895 | 0,66 | 430/80HS | 4,15:1 | 241 | VP10½S |  |  |  |  |
|  |  |  |  |  | 3,48:1 | 287 | VP10½S |  |  |  |  |
| ##### 5L20 | 1000 | 825/1120 | 0,825 | ###### 500HS/LS | 4,05:1 | 247 | VP11 |  |  |  |  |
| ##### 6L20 | 1000 | 990/1345 | 0,99 | 500HS | 4,05:1 | 247 | VP11 |  |  |  |  |
| ##### 8L20 | 1000 | 1320/1790 | 1,32 | 500/120HS | 4,24:1 | 236 | VP11½ |  |  |  |  |
| ##### 9L20 | 1000 | 1485/2020 | 1,485 | 500/120HS | 4,24:1 | 236 | VP11½ |  |  |  |  |
| 6L26 | 900 | 1860/2525 | 2,07 | 500/120HS | 3,32:1 | 271 | VP12 |  |  |  |  |
| 8L26 | 900 | 2480/3370 | 2,76 | 580HS | 4,15:1 | 217 | VP14 |  |  |  |  |
| ##### 9L26 | 900 | 2790/3790 | 3,1 | 650HS | 4,53:1 | 199 | VP14S |  |  |  |  |
| 12V26 | 900 | 3720/5055 | 4,13 | 720HS | 4,56:1 | 197 |  |  |  |  |  |
| ##### 16V26 | 900 | 4960/6740 | 5,51 | 800HS | 4,11:1 | 219 |  |  |  |  |  |
| 18V26 | 900 | 5580/7580 | 6,2 | 800HS | 3,62:1 | 248 |  |  |  |  |  |

Above is some indicative combinations, but please observe, that many other solutions are possible.

mekanord: Phone: + 45 96 20 14 00,  Fax: + 45 96 20 14 14,  e-mail:  mekanord@mekanord.dk                                     * = Theoretical

mekanord C.P.-propulsion 	Matching list for Wartsila -engines	15.11.00

| # Engine |  |  |  | # Gearbox |  | # C.P-propeller |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | # Open |  | # In Nozzle |  |
| Type | RPM | kW/HP | kW/RPM | Type | Red. ratio | RPM | #### Type | Diameter (m) | Bollard pull* (Tons) | Diameter (m) | Bollard pull* ### (Tons) |
| ##### 4L20 | 1000 | 660/895 | 0,66 | 430/80HS | 4,15:1 | 241 | VP10½S | 2,35 | 11,0 | 2,11 | 14,0 |
|  |  |  |  |  | 3,48:1 | 287 | VP10½S | 2,11 | 10,3 | 1,94 | 13,2 |
| ##### 5L20 | 1000 | 825/1120 | 0,825 | ###### 500HS/LS | 4,05:1 | 247 | VP11 | 2,42 | 13,1 | 2,21 | 16,7 |
| ##### 6L20 | 1000 | 990/1345 | 0,99 | 500HS | 4,05:1 | 247 | VP11 | 2,50 | 15,1 | 2,31 | 19,4 |
| ##### 8L20 | 1000 | 1320/1790 | 1,32 | 500/120HS | 4,24:1 | 236 | VP11½ | 2,73 | 19,4 | 2,54 | 25,1 |
| ##### 9L20 | 1000 | 1485/2020 | 1,485 | 500/120HS | 4,24:1 | 236 | VP11½ | 2,80 | 21,3 | 2,62 | 27,7 |
| ##### 6R25 | 720 | 1110/1500 | 1,54 | 500/120HS | 3,63:1 | 198 | VP11½/12 | 3,93 | 18,0 | 2,65 | 22,9 |
| 6R25 | 1000 | 1545/2100 | 1,545 | 500/120HS | 4,24:1 | 236 | VP12 | 2,82 | 22,0 | 2,64 | 28,6 |
| ##### 12V25 | 720 | 2210/3000 | 3,07 | 580HS | 3,48:1 | 207 | VP14 | 3,28 | 30,8 | 3,09 | 40,3 |
| 12V25 | 1000 | 2760/3750 | 2,76 | 580HS | 4,53:1 | 221 | VP14 | 3,30 | 35,9 | 3,16 | 47,4 |
| ##### 16V25 | 720 | 2940/4000 | 4,08 | 720HS | 3,58:1 | 201 | O.R. |  |  |  |  |
| ##### 16V25 | 1000 | 3680/5000 | 3,68 | 650HS | 4,05:1 | 247 | O.R. |  |  |  |  |
| 6L26 | 900 | 1860/2525 | 2,07 | 500/120HS | 3,32:1 | 271 | VP12 | 2,69 | 24,1 | 2,58 | 31,9 |
| 8L26 | 900 | 2480/3370 | 2,76 | 580HS | 4,15:1 | 217 | VP14 | 3,26 | 33,2 | 3,10 | 43,7 |
| ##### 9L26 | 900 | 2790/3790 | 3,1 | 650HS | 4,53:1 | 199 | O.R. |  |  |  |  |
| 12V26 | 900 | 3720/5055 | 4,13 | 720HS | 4,56:1 | 197 | O.R. |  |  |  |  |
| ##### 16V26 | 900 | 4960/6740 | 5,51 | 800HS | 4,11:1 | 219 | O.R. |  |  |  |  |
| 18V26 | 900 | 5580/7580 | 6,2 | 800HS | 3,62:1 | 248 | O.R. |  |  |  |  |

Above is some indicative combinations, but please observe, that many other solutions are possible.

mekanord: Phone: + 45 96 20 14 00,  Fax: + 45 96 20 14 14,  e-mail:  mekanord@mekanord.dk            O.R.= On Request                     * = Theoretical